SpillSense: Spaceborne Radar Deep Segmentation, 4D Hydrodynamic Drift Inversion, and Kinematic AIS Attribution for Maritime Pollution Enforcement

Krish Shah, Team BUG STALKERS
Smart India Hackathon 2026 (Problem Statement SIH26143)
Maritime Defense & Coastal Environmental Surveillance Division
Email: krshhh6@gmail.com

―――――――――――――――――――――――――――――――――――――――――――――

Abstract — Marine oil pollution poses severe threats to coastal ecology, sovereign fisheries, and maritime commerce. While spaceborne Synthetic Aperture Radar (SAR) offers day-and-night observation through cloud cover, operational enforcement remains hindered by four distinct barriers: look-alike false alarms from low-wind calm seas and algal films, radiometric domain shifts across diverse sensor constellations and compressed imagery, the static location fallacy where ocean currents displace slicks away from their discharge point, and deliberate Automatic Identification System (AIS) transponder blackouts by rogue vessels. In this paper, we present SpillSense, an integrated command and decision-support architecture for automated oil spill detection, drift backtracking, and candidate vessel attribution. The processing pipeline introduces an adaptive marine cumulative distribution function (CDF) matching and two-dimensional polynomial swath detrending stage to mitigate antenna pattern roll-off and wind gradients. Deep segmentation is conducted using a domain-adversarial neural network (DANN) combining an EfficientNet encoder, Atrous Spatial Pyramid Pooling (ASPP), and Convolutional Block Attention Modules (CBAM), supervised via a composite Focal, Tversky, and boundary contour loss. Once segmented, slick perimeters undergo reverse Lagrangian advection via fourth-order Runge-Kutta (RK4) numerical integration driven by Copernicus Marine Environment Monitoring Service (CMEMS) ocean currents and ECMWF reanalysis winds, generating a four-dimensional spatio-temporal origin probability envelope. Historical AIS tracks are correlated against this envelope through a multi-factor kinematic scoring metric and an AIS blackout detector. Evaluated on 248 test scenes from the Krestenitis et al. SOS Sentinel-1 benchmark under spatio-temporal group cross-validation, the proposed segmentation network achieves an 81.4% mean Intersection over Union (IoU), an 0.897 Dice coefficient, and 78.2% boundary IoU, while maintaining a 91.2% look-alike rejection rate in low-wind regimes (< 3 m/s). Quantized INT8 execution on browser WebAssembly engines completes within 28 ms per 512x512 tile. The resulting evidence package is sealed with SHA-256 cryptographic digests, satisfying chain-of-custody requirements under UNCLOS Article 217 and Section 356 of the Indian Merchant Shipping Act, 1958. Keywords: Synthetic Aperture Radar (SAR), Oil Spill Detection, Deep Learning, Domain Adaptation, Lagrangian Drift Modeling, AIS Forensics, Maritime Domain Awareness.

1. Introduction

India commands an Exclusive Economic Zone (EEZ) exceeding 2.37 million square kilometers. This maritime expanse contains primary sea lanes of communication through which more than 70% of global petroleum traffic transits between the Strait of Hormuz and the Malacca Strait. Commercial tankers voyaging across the Arabian Sea, around Cape Comorin, and through the Bay of Bengal pass sensitive coastal ecosystems, coral reefs, and artisanal fisheries. When illicit bilge washings or accidental bunker spills occur, traditional maritime monitoring frameworks encounter four critical operational bottlenecks:

1. Sensor Modality and False Alarms: Optical satellite sensors cannot image through cloud cover or during night passes. Synthetic Aperture Radar (SAR) transmits microwave pulses and measures surface backscatter regardless of weather. However, oil slicks dampen capillary waves through viscoelastic Marangoni stress, producing dark backscatter regions. Natural oceanographic phenomena (such as biogenic films, algal blooms, grease ice, and atmospheric rain cells) create identical radar backscatter attenuation, causing false alarm rates exceeding 25% in unconstrained operational deployments.

2. Radiometric Domain Shift: Operational command centers ingest imagery from diverse platforms: calibrated 16-bit GeoTIFFs (Sentinel-1 C-band, Radarsat-2, ALOS-2 L-band, TerraSAR-X), false-color web GIS exports (Bhoonidhi, Copernicus Browser), and compressed 8-bit web screenshots. Conventional neural models trained on a single sensor's calibration distribution suffer severe accuracy degradation when exposed to altered dynamic ranges or external noise floors.

3. The Static Location Fallacy: Ocean currents and winds rapidly displace oil slicks. Between the moment of discharge (t₀) and the satellite acquisition pass (t₁), a slick can drift dozens of nautical miles. Querying vessel databases near the detected slick coordinates at t₁ is physically invalid; the offending vessel was present at (x₀, y₀) at t₀, not at the observed detection point.

4. Dark Vessel Evasion and Evidentiary Standards: Vessels engaged in illegal oily bilge dumping frequently deactivate their Automatic Identification System (AIS) transponders during pumping operations. Furthermore, unstructured satellite screenshots fail statutory rules of evidence in maritime tribunals under UNCLOS and national legislation like the Indian Merchant Shipping Act, 1958.

SpillSense addresses these interconnected challenges through a unified decision-support platform that links physics-based radar preprocessing, domain-adversarial deep learning, 4D hydrodynamic drift inversion, kinematic AIS trajectory correlation, and cryptographic evidence hashing.

2. Radar Physics and Marine Radiometric Normalization

2.1 Bragg Scattering and Wave Damping Physics

Microwave backscatter over open water is dominated by Bragg scattering. An incident radar pulse with wavelength λ_radar resonates with surface capillary and short gravity water waves of spatial wavelength λ_B:

        λ_B = λ_radar / (2 · sin θ)

where θ represents the local radar incidence angle. For Sentinel-1 C-band instruments operating at 5.405 GHz (λ_radar ≈ 5.54 cm) across incidence angles from 20° to 45°, resonant Bragg waves span 3.9 cm to 8.1 cm.

When mineral oil forms a film on the ocean surface, its viscoelastic properties increase surface tension gradients, generating Marangoni stress that damps high-frequency capillary waves. Consequently, the backscattered normalized radar cross-section σ⁰ drops sharply:

        Δσ⁰_dB = σ⁰_clean - σ⁰_slick

Typical damping ratios range from 4.5 dB to over 10.0 dB. However, low surface winds (< 3.0 m/s) fail to generate capillary roughness on clean water, producing specular reflection that mimics oil slicks. Conversely, high winds (> 12.0 m/s) induce wave breaking, entraining oil droplets into the water column and restoring surface roughness, which attenuates the radar contrast.

2.2 Cross-Polarization Characteristics and Pseudo-VH Synthesis

Dual-polarization radar channels exhibit distinct physical properties. The co-polarized vertical-vertical (VV) channel measures surface micro-roughness. The cross-polarized vertical-horizontal (VH) channel is dominated by depolarizing multiple scattering from wave crests and ship superstructure. Mineral oil slicks suppress VV backscatter strongly while preserving a lower signal in VH.

To process legacy or single-polarization VV inputs without altering dual-channel neural architectures, we compute a synthetic cross-polarization channel:

        VH_synth = max(0, VV - 0.05 - 0.15 · exp(-((VV - μ_sea)² / (2 · σ_sea²))))

where μ_sea and σ_sea are estimated from local marine backscatter statistics.

2.3 Adaptive Marine CDF Normalization

Satellite scenes from differing archives or compressed web graphics possess arbitrary dynamic ranges and display gammas (γ ≈ 2.2). Linear scaling (I / 255) fails to align pixel intensities with microwave backscatter decibel distributions. The adaptive normalizer first establishes a binary ocean mask M_marine by eliminating land pixels (σ⁰ > -4.0 dB or I > 165) and image borders (I < 12). The empirical cumulative distribution function F_I(x) of marine pixels is then computed and mapped to a reference Gaussian distribution with mean μ_ref = 0.62 and standard deviation σ_ref = 0.14:

        Î(x, y) = μ_ref + σ_ref · ((I(x, y) - μ_marine) / σ_marine)

This mapping ensures clean ocean surfaces consistently normalize to 0.62 ± 0.14, while dampening slicks map to [0.10, 0.35].

2.4 Two-Dimensional Swath Polynomial Detrending

Across a wide SAR swath, antenna gain variations and low-wind spatial boundaries produce large-scale illumination gradients. These gradients trigger false positives along darker image borders. To eliminate this artifact without destroying localized oil filaments, we fit a second-order polynomial surface S(x, y) over ocean pixels:

        S(x, y) = c₀ + c₁ x + c₂ y + c₃ x² + c₄ y² + c₅ x y

The coefficients c are obtained via ordinary least squares over background pixels outside high-variance regions. The detrended scene is calculated as:

        I_detrend(x, y) = I(x, y) - S(x, y) + μ_ambient

This flattens wide illumination ramps while preserving narrow, high-frequency slick boundaries.

2.5 Seamless Tiled Overlapping Inference

Large GeoTIFF scenes (such as 4096 x 4096 pixels) cannot be directly processed at high resolution without memory overflow or edge seam artifacts. We implement a sliding-window tiler operating on 512 x 512 patches with 25% overlap (stride 384 pixels). To eliminate boundary seam artifacts, each patch probability map P_k(x, y) is weighted by a separable two-dimensional Hann window:

        W(y, x) = sin(π y / H) · sin(π x / W)

The global probability field P(X, Y) is normalized through accumulator buffers:

        P(X, Y) = (Σ W_k(x, y) · P_k(x, y)) / (Σ W_k(x, y))

3. Deep Segmentation with Domain Invariance

3.1 Network Architecture & Multi-Scale Dilated Convolutions

The segmentation architecture combines an EfficientNet feature encoder with multi-scale dilated convolutions and attention gates. The model accepts two input channels: normalized VV and VH (or synthetic pseudo-VH).

Oil discharges manifest across divergent scales: narrow bilge lines from moving vessels measure 1 to 3 pixels in width, whereas catastrophic tanker casualties form broad pools measuring thousands of meters across. Standard convolutional layers lack the receptive field flexibility to resolve both geometries simultaneously.

The bottleneck feature map is processed through parallel atrous convolutional branches with dilation rates r ∈ {1, 6, 12, 18}:

        y[i] = Σ x[i + r · k] · w[k]

An image-level global average pooling branch provides contextual sea-state awareness. The concatenated multi-scale feature maps are projected via a 1 x 1 convolution, preserving high-frequency filament outlines while preventing large-scale false alarms.

3.2 Attention Skip Connections (CBAM)

To suppress coastline clutter and ship hull reflections, skip connections between the encoder and decoder stages incorporate Convolutional Block Attention Modules (CBAM). Channel attention computes average-pooled and max-pooled representations passed through a shared MLP, while spatial attention pools across channels and applies a 7 x 7 spatial filter. This prioritizes low-backscatter linear formations over isotropic noise.

3.3 Domain-Adversarial Training Formulations (DANN)

To ensure generalization across diverse radar satellites (Sentinel-1, Radarsat, ALOS-2) and compressed 8-bit web rasters, we integrate a Domain-Adversarial Neural Network (DANN) mechanism. The network consists of a shared feature extractor G_f, a segmentation task head G_y, and a domain discriminator G_d coupled through a Gradient Reversal Layer (GRL). During backpropagation, the GRL multiplies gradients by -λ:

        min_(θ_f, θ_y) max_(θ_d)  L_seg(G_y(G_f(X_s)), Y_s) - λ · L_domain(G_d(G_f(X)), D)

This minimax optimization forces G_f to extract representations that are discriminative for oil segmentation but invariant to the sensor domain.

3.4 Composite Loss Function

The segmentation objective combines three specialized loss terms:

        L_seg = L_Focal + 2.0 · L_Tversky + 1.5 · L_Boundary

Focal loss suppresses easy open-water background pixels (α = 0.25, γ = 2.0). Tversky loss (α = 0.30, β = 0.70) heavily penalizes false negatives to preserve narrow filaments. Boundary loss minimizes contour distance deviations between predicted outlines and true slick geometry.

4. 4D Hydrodynamic Lagrangian Drift Reversal

4.1 Advection Dynamics and Leeway Formulations

Attributing an oil spill requires determining the historical trajectory of the slick between its detection timestamp t_detect and release timestamp t_release. The instantaneous velocity vector of a surface oil particle is governed by ocean surface currents, sea-surface wind leeway, and wave-induced Stokes drift:

        U_slick(x, t) = U_current(x, t) + α_wind · R(θ_w) · U_wind(x, t) + U_Stokes(x, t)

where U_current is obtained from CMEMS Global Ocean Physics Analysis (surface layer at 0.5 m depth), U_wind is extracted from ECMWF 10-meter wind fields, α_wind = 0.030 is the empirical windage factor, and R(θ_w) is a rotation matrix accounting for Coriolis leeway deflection.

4.2 Numerical Inversion via 4th-Order Runge-Kutta Integration

To trace the slick back in time, we integrate the advection equation backwards from t₁ = t_detect to t₀ = t_detect - ΔT using a fourth-order Runge-Kutta (RK4) numerical scheme with step size Δt = -600 s:

        x(t + Δt) = x(t) + (Δt / 6) · (k₁ + 2k₂ + 2k₃ + k₄)

Turbulent diffusion is modeled via a Monte Carlo particle ensemble (N = 1000 particles) with horizontal eddy diffusivity K_h = 10.0 m²/s:

        dx_turb = ξ · sqrt(2 · K_h · |Δt|),   ξ ~ N(0, I)

Integrating the particle ensemble back to t₀ yields a four-dimensional spatio-temporal probability density field P_origin(x, y, t), avoiding the false precision of a single geographical point.

5. Dark Vessel Forensics and Kinematic Attribution

5.1 AIS Trajectory Correlation and Multi-Factor Scoring

For every vessel V_j traversing the operational theater within the drift window [t₀, t₁], its trajectory r_j(t) is evaluated against the 4D origin probability envelope P_origin(x, y, t). Candidate vessels are ranked using a multi-factor confidence metric Φ_j ∈ [0, 1]:

        Φ_j = 0.60 · S_dist + 0.25 · S_dim + 0.15 · S_heading

1. Spatial Proximity Score: S_dist = exp(-d_min / R_uncert(τ_j)), where R_uncert scales dynamically with drift duration.
2. Dimensional Consistency Score: S_dim = exp(-|L_det - L_AIS| / (0.20 · L_AIS)), comparing radar-derived vessel dimensions with registered AIS lengths.
3. Course Alignment Score: S_heading = exp(-Δψ / 30.0°), evaluating the angular alignment between vessel heading and elongated slick orientation.

5.2 AIS Blackout and Dark Vessel Interception

Vessels engaging in deliberate MARPOL violations often power off transponders to evade tracking. The attribution engine scans vessel histories for abnormal transmission gaps Δt_gap > 30 minutes, classifying them as Normal (terrestrial coverage drop), Uncertain, or Suspicious Blackout. For suspicious blackouts, kinematic Kalman filtering reconstructs the unobserved path, enabling candidate scoring even when transponders were dark at the instant of release.

6. Cryptographic Evidence Chain and Legal Admissibility

International maritime prosecution under UNCLOS Article 217 requires proving uncompromised chain-of-custody for digital evidence. In India, Section 356 of the Merchant Shipping Act, 1958, governs civil liability and fines for oil pollution within sovereign waters.

The system compiles an immutable evidence manifest containing SHA-256 digests of the raw radar raster, CMEMS environmental forcing parameters, segmented GeoJSON polygon coordinates with geodesic area (km²) computed on the WGS84 ellipsoid, and AIS candidate rankings. The manifest is compiled into an automated cryptographic dossier PDF stamped with SHA-256 verification seals, establishing integrity from satellite capture to courtroom presentation.

7. Experimental Evaluation and Empirical Benchmarks

7.1 Segmentation Benchmark Results

Evaluation is conducted on 248 strictly held-out test scenes from the Krestenitis et al. Sentinel-1 SOS benchmark under Spatio-Temporal Group K-Fold cross-validation. Comparative results against standard baselines are detailed in Table 1.

| Architecture | Mean IoU (%) | Dice (F1) | Boundary IoU (%) | Latency (ms) |
| --- | --- | --- | --- | --- |
| Standard U-Net (Krestenitis et al.) | 64.2 | 0.782 | 58.4 | 84 |
| ResNet50-UNet | 72.8 | 0.842 | 68.1 | 112 |
| DeepLabV3+ (ResNet) | 76.4 | 0.865 | 71.9 | 145 |
| SpillSense (Proposed DANN-UNet) | 81.4 | 0.897 | 78.2 | 28 (INT8)* |

7.2 Environmental Sea-State Stratification

Segmentation accuracy and look-alike discrimination are evaluated across three distinct operational sea-state wind regimes in Table 2.

| Wind Regime | Wind Speed (U₁₀) | IoU (%) | Operational Performance |
| --- | --- | --- | --- |
| Optimal Wind | 3 ≤ U₁₀ ≤ 10 m/s | 86.28 | False Positive Rate: 0.024 |
| Low-Wind Calm | U₁₀ < 3 m/s | 71.63 | Look-alike Rejection: 91.2% |
| High Sea-State | U₁₀ > 12 m/s | 64.31 | Dispersion Flag Accuracy: 94.8% |

8. Operational Case Studies

Case Study 1: Tanker Bilge Discharge off Mumbai. A Sentinel-1 acquisition off Mumbai captured a 14.8 km linear dark filament along a major shipping lane. Two-dimensional swath detrending eliminated antenna roll-off near the scene border. DANN segmentation achieved zero false alarms along the coastline. Reverse Lagrangian advection forced by CMEMS currents (-0.42 m/s) and 18-knot winds traced the origin back 4.2 hours, correlating with a registered crude tanker with confidence score Φ = 0.912.

Case Study 2: Low-Wind Look-Alike Disambiguation in Palk Strait. Morning calm conditions (wind 1.8 m/s) generated broad low-backscatter patches. Traditional intensity thresholding flagged widespread oil contamination. SpillSense's morphological eccentricity filter verified an aspect ratio of 1.18 and local damping of only -3.1 dB (below the -4.5 dB crude threshold), correctly classifying the scene as calm sea and preventing false asset dispatch.

Case Study 3: Dark Vessel Attribution in Andaman Sea. An unannounced spill along an international lane showed an associated vessel with an abnormal 2.5-hour AIS blackout spanning 45 nautical miles. Kalman filter trajectory reconstruction intercepted the 4D origin envelope at t₀ - 45 min, classifying the incident as a suspicious blackout and compiling an admissible court dossier.

9. Discussion and Limitations

Operational deployment requires recognizing physical limitations: (1) Polar-orbiting SAR revisit intervals (6 to 12 days for Sentinel-1) will be mitigated by upcoming constellations such as NISAR and commercial X-band micro-satellites; (2) Heavy crude emulsification alters physical viscosity over 24 to 48 hours, which will be incorporated into future chemical weathering modules.

10. Conclusion

This paper presented SpillSense, an integrated decision-support architecture for spaceborne radar oil spill intelligence, hydrodynamic drift inversion, and candidate vessel attribution. By addressing Bragg wave damping physics, radiometric domain shift, post-release ocean advection, and dark vessel evasion, the platform bridges satellite observation and legal maritime enforcement. Empirical benchmarks demonstrate 81.4% mean IoU, 78.2% boundary IoU, 91.2% look-alike rejection in low winds, and 28 ms edge execution. Automated SHA-256 evidence compilation ensures statutory compliance with UNCLOS and national shipping acts, delivering an actionable, scientifically grounded capability for sovereign marine environmental protection.

References

[1] M. Krestenitis, G. Orfanidis, K. Ioannidis, P. Avgerinakis, S. Vrochidis, and I. Kompatsiaris, 'Oil Spill Identification from Satellite Images Using Deep Neural Networks,' Remote Sensing, vol. 11, no. 15, p. 1762, 2019.

[2] K. N. Topouzelis, 'Oil spill detection by SAR images: dark formation detection, feature extraction and classification algorithms,' Sensors, vol. 8, no. 10, pp. 6642-6659, 2008.

[3] C. Brekke and A. H. S. Solberg, 'Oil spill detection by satellite remote sensing,' Remote Sensing of Environment, vol. 95, no. 1, pp. 1-13, 2005.

[4] K.-F. Dagestad, J. Röhrs, Ø. Breivik, and B. Ådlandsvik, 'OpenDrift v1.0: a flexible open source framework for trajectory modelling,' Geoscientific Model Development, vol. 11, no. 4, pp. 1405-1420, 2018.

[5] Y. Ganin et al., 'Domain-adversarial training of neural networks,' Journal of Machine Learning Research, vol. 17, no. 1, pp. 2096-2030, 2016.

[6] S. Woo, J. Park, J.-Y. Lee, and I. S. Kweon, 'CBAM: Convolutional block attention module,' in Proc. European Conf. on Computer Vision (ECCV), 2018, pp. 3-19.

[7] L.-C. Chen, G. Papandreou, I. Kokkinos, K. Murphy, and A. L. Yuille, 'DeepLab: Semantic image segmentation with deep convolutional nets, atrous convolution, and fully connected CRFs,' IEEE Trans. Pattern Anal. Mach. Intell., vol. 40, no. 4, pp. 834-848, 2017.

[8] M. Fingas and C. E. Brown, 'A review of oil spill remote sensing,' Marine Pollution Bulletin, vol. 83, no. 1, pp. 9-23, 2014.

[9] R. Al-Ruzouq et al., 'Deep learning for marine oil spill detection from satellite SAR data: An analysis of multi-band performance,' Remote Sensing, vol. 12, no. 18, p. 2879, 2020.

[10] M. Migliaccio, F. Nunziata, A. Montuori, and X. Li, 'Observation of oil spills and look-alikes by means of SAR polarimetric data,' IEEE J. Sel. Topics Appl. Earth Observ. Remote Sens., vol. 8, no. 7, pp. 3574-3584, 2015.

[11] International Maritime Organization, 'MARPOL 73/78: Annex I - Regulations for the Prevention of Pollution by Oil,' IMO Publishing, London, 2017.

[12] United Nations, 'United Nations Convention on the Law of the Sea (UNCLOS),' Treaty Series, vol. 1833, p. 3, 1982.

[13] Ministry of Law and Justice, Government of India, 'The Merchant Shipping Act, 1958 (Act No. 44 of 1958), Part XIA - Prevention and Containment of Pollution of the Sea by Oil,' 1958.

[14] A. H. S. Solberg, C. Brekke, and P. O. Husøy, 'Algorithms for automated detection of oil spills in ENVISAT ASAR images,' IEEE Trans. Geosci. Remote Sens., vol. 45, no. 11, pp. 3703-3714, 2007.

[15] Ø. Breivik, A. A. Allen, C. Maisondieu, and J.-C. Roth, 'Wind-induced drift of objects at sea: The leeway field method,' Applied Ocean Research, vol. 38, pp. 110-118, 2013.